Model No		: KX-VC1600/VC1300/VC1000

Subject		: Remote Control App for Android

Effective from	: N/A

Countries/Areas	: All

Type  		:   Mandatory    Preventive    Optional    Information only    Parts Change  	  Notice

Recommend 	:   At Setup      At Occurrence    Next Visit    Next PM

Please be informed that we have developed “Remote Control App for Android”.

The App. can be used to control HDVC codec via external control API from a remote site. This is one of sales promotion material with Free of Charge for the purpose of sales expansion. Therefore, you can use with your own responsibility, and PSN doesn’t guarantee the operation, specification, performance at all.

It is available through the BIG-WEB and Panasonic B2B Extranet.

https://bizpartner.panasonic.net/extranet/product/

You can pass this application to your sincere partners and end users.

Please note that this application won't be on google play. Also it runs on Android OS version 4.4 or later.

The model and software version of HDVC that the App. can control are following,

- KX-VC1600/VC1300 (Version 4.25 or later)

- KX-VC1000 (Version 4.30 or later)

- System Structure

- Installation

This App. cannot be downloaded from Google Play. Application package file (.apk) should be stored in the Android terminal and install locally.

Procedure:

1. Connect Android terminal to the PC with USB cable

2. Mount to push “USB connection” in the Android terminal

3. Confirm the Drive to access the Android terminal in the PC

4. Copy to the apk file from the PC to the Android terminal

5. Release to mount the USB connection in the Android terminal

6. Start up some kind of File Manager Application, and select the apk file and click to start installation

7. After finishing the installation, click the icon (named “HdvcRemote”) to start the App.

Note1: You have to confirm the end user licenses agreement carefully when you install at first.

Note2: The HDVC should be set the web port (port 80) open as always to use this application.